CMPEN 331 Final Project

Section 1

Victor Xu

12/17/25

Dr. Mohamed Almekkawy

# Abstract

In this final project for CMPEN 331, I am adding hazard detection capabilities to a data path consisting of 5 stages: instruction fetch, instruction decode, execute, memory access, and write back. The detection logic identifies data hazards that occur between dependent instructions. In addition, I am adding forwarding modules that enable operands from earlier instructions to the more recent instructions without waiting for write-back. These improvements reduce unnecessary stalls and ensure proper execution of dependent instructions.

# Introduction

Sometimes when people are in a hurry, they brush their teeth in the shower. Or they eat their breakfast while driving. Or they borrow the work done by a colleague. This sort of behavior is like pipelining that allows for overlapping execution in modern processors. Rather than waiting for the first instruction’s completion before the next, a processor with pipelined architecture divides execution into stages allowing instructions to be processed simultaneously at different stages of execution. This project implements a five-stage pipelined Datapath consisting of Instruction Fetch (FD), Instruction Decode (ID) , Execute (EXE), Memory Access (MEM).

Figure 1;  5-stage Datapath with no Hazard Detection

The IF stage gets the instruction from the memory via the PC. The PC value is then increased to prepare for the next cycle. This instruction, through a register, is passed to ID stage which, through its control unit, translates what the instruction value wants to do.  It also reads values from the register file. Control signals and these register values are then passed to the EXE stage through another register. In the EXE stage, the Arithmetic Logic Unit (ALU) performs whatever operation was decoded from the ID stage. The result and the data to be stored are passed to the MEM stage. The MEM stage then reads or writes to the data memory depending on what the control signals from the ID stage dictate. Sometimes this stage does nothing for operations like “add”. The data read from memory is then passed to the WB stage. Depending on the instruction, the WB stage selects between the memory data and ALU result to write back the result into the destination register in the register file.

Figure 2; Several instructions in sequence demonstrating data forwarding

However, pipelining comes with its dangers. When instructions depend on previous instructions that have yet to be complete, dependencies create data hazards where the data from one instruction is not ready when it is needed for another. The solution for this issue is hazard detection and data forwarding.  Hazard detection logic identifies cases where there is overlap in past and current instructions. The forwarding unit goes off this logic to then directly route results from later pipeline stages to earlier ones.

# Verilog Design Code

`timescale 1ns / 1ps

//////////////////////////////////////////////////////////////////////////////////

// Company:

// Engineer:

//

// Create Date: 11/03/2025 10:27:38 AM

// Design Name:

// Module Name: TopModule

// Project Name:

// Target Devices:

// Tool Versions:

// Description: CMPEN 331 Lab 3 - IF and ID Pipeline Implementation

//

//////////////////////////////////////////////////////////////////////////////////

module TopModule(

input wire clock,

output wire [31:0] wbData

);

// IF/ID outputs

wire[31:0] pc;

wire [31:0] dinstOut;

// ID/EXE outputs

wire ewreg, em2reg, ewmem;

wire [3:0] ealuc;

wire ealuimm;

wire [4:0] edestReg;

wire [31:0] eqa, eqb, eimm32;

// EXE/MEM outputs

wire mwreg, mm2reg, mwmem;

wire [4:0] mdestReg;

wire [31:0] mr, mqb;

// MEM/WB outputs

wire wwreg, wm2reg;

wire [4:0] wdestReg;

wire [31:0] wr, wdo;

wire [31:0] wbData;

wire [31:0] pc_next;

wire [31:0] pc_current;

wire [31:0] out_inst;

wire [31:0] out_dinst;

ProgramCounter PC_inst (

.clock  (clock),

.nextPc (pc_next),

.pc     (pc_current)

);

Adder Add_inst (

.pc     (pc_current),

.nextPc (pc_next)

);

InstructionMemory InstructionMem_inst (

.pc      (pc_current),

.instOut (out_inst)

);

IF_ID_Register IFID_inst (

.clock    (clock),

.instOut  (out_inst),

.dinstOut (out_dinst)

);

// Field extraction

wire [5:0]  inst_op   = out_dinst[31:26];

wire [5:0]  inst_func = out_dinst[5:0];

wire [4:0]  inst_rs   = out_dinst[25:21];

wire [4:0]  inst_rt   = out_dinst[20:16];

wire [4:0]  inst_rd   = out_dinst[15:11];

wire [15:0] inst_imm  = out_dinst[15:0];

// Control signals

wire out_wreg;

wire out_m2reg;

wire out_wmem;

wire [3:0] out_aluc;

wire out_aluimm;

wire out_regrt;

wire [1:0] fwdaw;

wire [1:0] fwdbw;

ControlUnit Control_inst (

.op     (inst_op),

.func   (inst_func),

.rs(inst_rs),

.rt(inst_rt),

.ern(edestRegw),

.mrn(mdestRegw),

.mm2reg(mm2regw),

.mwreg(mwregw),

.em2reg(out_m2reg),

.ewreg(out_wreg),

.wreg   (out_wreg),

.m2reg  (out_m2reg),

.wmem   (out_wmem),

.aluc   (out_aluc),

.aluimm (out_aluimm),

.regrt  (out_regrt),

.fwda(fwdaw),

.fwdb(fwdbw)

);

wire [31:0] MUXaw;

wire [31:0] MUXbw;

fwdMux fwdMuxA (

.q(out_qa),

.r(ALU_r),

.mr(mrw),

.do(mdow),

.select(fwdaw),

.fwd(MUXaw)

);

fwdMux fwdMuxB (

.q(out_qb),

.r(ALU_r),

.mr(mrw),

.do(mdow),

.select(fwdbw),

.fwd(MUXbw)

);

// regrt mux (RegDst)

wire [4:0] out_destReg;

Mux_RegDst MuxRegDst_inst (

.rt      (inst_rt),

.rd      (inst_rd),

.regrt   (out_regrt),

.destReg (out_destReg)

);

// Register File

wire [31:0] out_qa, out_qb;

RegisterFile Register_inst (

.rs (inst_rs),

.rt (inst_rt),

.wdestReg(wdestRegw),

.wbData(wbDataw),

.wwreg(wwregw),

.clk(clock), //clock signal is inverted within register

.qa (out_qa),

.qb (out_qb)

);

// Sign Extension

wire [31:0] out_imm32;

SExtend Extend_inst (

.imm   (inst_imm),

.imm32 (out_imm32)

);

wire ealuimmw;

wire [31:0] eqaw;

wire [31:0] eqbw;

wire [31:0] eimm32w;

wire [3:0] ealucw;

wire ewregw;

wire em2regw;

wire ewmemw;

wire [4:0] edestRegw;

ID_EXE_Register IDEXE_inst (

.wreg     (out_wreg),

.m2reg    (out_m2reg),

.wmem     (out_wmem),

.aluc     (out_aluc),

.aluimm   (out_aluimm),

.destReg  (out_destReg),

.qa       (MUXaw),

.qb       (MUXbw),

.imm32    (out_imm32),

.clock    (clock),

.ewreg    (ewregw),

.em2reg   (em2regw),

.ewmem    (ewmemw),

.ealuc    (ealucw),

.ealuimm  (ealuimmw),

.edestReg (edestRegw),

.eqa      (eqaw),

.eqb      (eqbw),

.eimm32   (eimm32w)

);

assign ealuimm = ealuimmw;

assign eqa = eqaw;

assign eqb = eqbw;

assign eimm32 = eimm32w;

assign ealuc = ealucw;

assign ewreg = ewregw;

assign em2reg = em2regw;

assign ewmem = ewmemw;

assign edestReg = edestRegw;

wire [31:0] mux_alu_b;

Mux_ALU Mux_ALU_inst (

.eqb (eqbw),

.eimm32 (eimm32w),

.ealuimm (ealuimmw),

.b (mux_alu_b)

);

wire [31:0] ALU_r;

ALU ALU_inst (

.eqa (eqaw),

.b (mux_alu_b),

.ealuc (ealucw),

.r (ALU_r)

);

wire [4:0] mdestRegw;

wire [31:0] mrw;

wire [31:0] mqbw;

wire mwregw;

wire mm2regw;

wire mwmemw;

EXE_MEM_Register EXEMEM_inst (

.ewreg (ewregw),

.em2reg (em2regw),

.ewmem (ewmemw),

.clk (clock),

.edestReg (edestRegw),

.r (ALU_r),

.eqb (eqbw),

.mwreg (mwregw),

.mm2reg (mm2regw),

.mwmem  (mwmemw),

.mdestReg (mdestRegw),

.mr (mrw),

.mqb (mqbw)

);

assign mwreg = mwregw;

assign mm2reg = mm2regw;

assign mwmem = mwmemw;

assign mdestReg = mdestRegw;

assign mr = mrw;

assign mqb = mqbw;

wire [31:0] mdow;

DataMemory DataMemory_inst(

.mr (mrw),

.mqb (mqbw),

.mwmem (mwmemw),

.clk (clock),

.mdo (mdow)

);

wire [31:0] wrw;

wire [31:0] wdow;

wire [4:0] wdestRegw;

wire wwregw;

wire wm2regw;

MEM_WB_Register MEMWB_inst (

.mwreg (mwregw),

.mm2reg (mm2regw),

.clk (clock),

.mr (mrw),

.mdo (mdow),

.mdestReg (mdestRegw),

.wwreg (wwregw),

.wm2reg (wm2regw),

.wdestReg (wdestRegw),

.wr (wrw),

.wdo (wdow)

);

assign dinstOut = out_dinst;

assign pc       = pc_current;

assign wr = wrw;

assign wdo = wdow;

assign wm2reg = wm2regw;

assign wwreg = wwregw;

assign wdestReg = wdestRegw;

wire [31:0] wbDataw;

WbMux WbMuxInst (

.wr(wrw),

.wdo(wdow),

.wm2reg(wm2regw),

.wbData(wbDataw)

);

assign wbData = wbDataw;

endmodule

module ProgramCounter(

input clock,

input [31:0] nextPc,

output reg [31:0] pc

);

initial begin

pc = 32'd100;

end

always @(posedge clock) begin

pc <= nextPc;

end

endmodule

module Adder(

input [31:0] pc,

output [31:0] nextPc

);

assign nextPc = pc + 4;

endmodule

module InstructionMemory(

input [31:0] pc,

output reg [31:0] instOut

);

reg [31:0] memory [0:63];

initial begin

memory[25] = 32'h00221820; //add $3, $1, $2

memory[26] = 32'h01232022; //sub $4, $9, $3

memory[27] = 32'h00692825; //or $5, $3, $9

memory[28] = 32'h00693026; //xor $6, $3, $9

memory[29] = 32'h00693824; //and $7, $3, $9

end

always @(*) begin

instOut <= memory[pc[7:2]]; //Only need 6 bits since we only have 64 wods in memory

end

endmodule

`timescale 1ns / 1ps

module ControlUnit(

input [5:0] op, func,

//new inputs for hazard detection and forwarding

input [4:0] rs, rt,

input [4:0] ern, mrn,

input mm2reg, mwreg, em2reg, ewreg,

output reg wreg,

output reg m2reg,

output reg wmem,

output reg [3:0] aluc,

output reg aluimm,

output reg regrt,

//new outputs for hazard detections and worwarding

output reg [1:0] fwda, fwdb

);

always @(*) begin

//defaults

wreg  = 1'b0;

m2reg = 1'b0;

wmem  = 1'b0;

aluc  = 4'b0010; // default to ADD

aluimm= 1'b0;

regrt = 1'b0;

fwda = 2'b00;

fwdb = 2'b00;

case(op)

6'b000000: //R-Format

begin

wreg = 1'b1;

m2reg = 1'b0;

wmem = 1'b0;

aluimm = 1'b0;

regrt = 1'b0; //rd

case (func)

6'b100000: //ADD Instruction

aluc = 4'b0010;

6'b100010: //SUB instruction

aluc = 4'b0110;

6'b100100: //AND instruction

aluc = 4'b0000;

6'b100101: //OR instruction

aluc = 4'b0001;

6'b100110: //XOR instruction

aluc = 4'b0011;

endcase

end

6'b100011: //lw

begin

wreg = 1'b1;

m2reg = 1'b1;

wmem = 1'b0;

aluimm = 1'b1;

regrt = 1'b1; //rt

aluc = 4'b0010;

end

endcase

//00 -> no hazards, forward qa or qb depending on multiplexer

//01 -> forward r

//10 -> forward mr

//11 -> forward do

//fwrda

if (mwreg && (mrn != 5'b00000) && (mrn == rs)) begin

fwda = (mm2reg) ? 2'b11 : 2'b10;

end

else if (ewreg && (ern != 5'b00000) && (ern == rs)) begin

fwda = 2'b01;

end

//fwrdb

if (mwreg && (mrn != 5'b00000) && (mrn == rt)) begin

fwdb = (mm2reg) ? 2'b11 : 2'b10;

end

else if (ewreg && (ern != 5'b00000) && (ern == rt)) begin

fwdb = 2'b01;

end

end

endmodule

module Mux_RegDst(

input [4:0] rt,

input [4:0] rd,

input regrt,

output [4:0] destReg

);

assign destReg = (regrt) ? rt : rd;

endmodule

module RegisterFile(

input [31:0] wbData,

input [4:0] rs,rt,wdestReg,

input wwreg, clk,

output reg [31:0] qa, qb

);

reg [31:0] regs [0:31];

initial begin: init_regs

integer i;

for (i=0; i < 32; i = i + 1)

regs[i] = 0;

end

always @(*) begin

if (rs == 5'd0)

qa = 32'b0;

else

qa = regs[rs];

if (rt == 5'd0)

qb = 32'b0;

else

qb = regs[rt];

end

always @(negedge clk) begin

if (wwreg)

regs[wdestReg] = wbData;

end

endmodule

module SExtend(

input [15:0] imm,

output reg [31:0] imm32

);

always @(*) begin

imm32 = {{16{imm[15]}}, imm};

end

endmodule

module ID_EXE_Register(

input wreg,

input m2reg,

input wmem,

input [3:0] aluc,

input aluimm,

input [4:0] destReg,

input [31:0] qa,

input [31:0] qb,

input [31:0] imm32,

input clock,

output reg ewreg,

output reg em2reg,

output reg ewmem,

output reg [3:0] ealuc,

output reg ealuimm,

output reg [4:0] edestReg,

output reg [31:0] eqa,

output reg [31:0] eqb,

output reg [31:0] eimm32

);

always @(posedge clock) begin

ewreg <= wreg;

em2reg <= m2reg;

ewmem <= wmem;

ealuc <= aluc;

ealuimm <= aluimm;

edestReg <= destReg;

eqb <= qb;

eqa <= qa;

eimm32 <= imm32;

end

endmodule

module Mux_ALU(

input [31:0] eqb, eimm32,

input ealuimm,

output reg [31:0] b

);

always @(*) begin

case (ealuimm)

1'b0: b = eqb;

1'b1: b = eimm32;

endcase

end

endmodule

module ALU(

input [31:0] eqa, b,

input [3:0] ealuc,

output reg [31:0] r

);

always @(*) begin

case (ealuc)

4'b0010: r = eqa + b; //ADD

4'b0110: r = eqa - b; //SUB

4'b0000: r = eqa & b; //AND

4'b0001: r = eqa | b; //OR

4'b0111: r = (eqa < b) ? 32'd1 : 32'd0; //SLT

default: r = 32'b0; //safety

endcase

end

endmodule

module EXE_MEM_Register(

input ewreg, em2reg, ewmem, clk,

input [4:0] edestReg,

input [31:0] r, eqb,

output reg mwreg, mm2reg, mwmem,

output reg [4:0] mdestReg,

output reg [31:0] mr, mqb

);

always @(posedge clk) begin

mwreg <= ewreg;

mm2reg <= em2reg;

mwmem <= ewmem;

mdestReg <= edestReg;

mr <= r;

mqb <= eqb;

end

endmodule

module DataMemory(

input [31:0] mr, mqb,

input mwmem, clk,

output reg [31:0] mdo

);

reg [31:0] memory [0:63];

initial begin

memory[0] = 32'hA00000AA;

memory[1] = 32'h10000011;

memory[2] = 32'h20000022;

memory[3] = 32'h30000033;

memory[4] = 32'h40000044;

memory[5] = 32'h50000055;

memory[6] = 32'h60000066;

memory[7] = 32'h70000077;

memory[8] = 32'h80000088;

memory[9] = 32'h90000099;

end

always @(*) begin

mdo = memory[mr[7:2]];

end

always @(negedge clk) begin

if (mwmem)

memory[mr[7:2]] <= mqb;

end

endmodule

module MEM_WB_Register(

input mwreg, mm2reg, clk,

input [4:0] mdestReg,

input [31:0] mr, mdo,

output reg wwreg, wm2reg,

output reg [4:0] wdestReg,

output reg [31:0] wr, wdo

);

always @(posedge clk) begin

wwreg <= mwreg;

wm2reg <= mm2reg;

wdestReg <= mdestReg;

wr <= mr;

wdo <= mdo;

end

endmodule

module WbMux(

input [31:0] wr, wdo,

input wm2reg,

output reg [31:0] wbData

);

always @(*) begin

wbData = wm2reg ? wdo:wr;

end

endmodule

module fwdMux(

input [31:0] q, //can be qa or qb

input [31:0] r, mr, do,

input [1:0] select,

output reg [31:0] fwd

);

always @(*) begin

case(select)

00: fwd = q;

01: fwd = r;

10: fwd = mr;

11: fwd = do;

endcase

end

endmodule

# Testbench Code

`timescale 1ns / 1ps

module testbench;

reg clock;

// IF/ID

wire [31:0] pc, dinstOut;

// ID/EXE

wire ewreg, em2reg, ewmem;

wire [3:0] ealuc;

wire ealuimm;

wire [4:0] edestReg;

wire [31:0] eqa, eqb, eimm32;

// EXE/MEM

wire mwreg, mm2reg, mwmem;

wire [4:0] mdestReg;

wire [31:0] mr, mqb;

// MEM/WB

wire wwreg, wm2reg;

wire [4:0] wdestReg;

wire [31:0] wr, wdo;

//FWD

wire [1:0] fwda;

wire [1:0] fwdb;

wire [31:0] qa;

wire [31:0] qb;

wire [31:0] MUXaw;

wire [31:0] MUXbw;

wire [31:0] wbData;

// DUT

TopModule dut(

.clock(clock),

.wbData(wbData)

);

assign pc = dut.pc;

assign dinstOut = dut.dinstOut;

assign ewreg = dut.ewreg;

assign em2reg = dut.em2reg;

assign ewmem = dut.ewmem;

assign ealuc = dut.ealuc;

assign ealuimm = dut.ealuimm;

assign edestReg = dut.edestReg;

assign eqa = dut.eqa;

assign eqb = dut.eqb;

assign eimm32 = dut.eimm32;

assign mwreg = dut.mwreg;

assign mm2reg = dut.mm2reg;

assign mwmem = dut.mwmem;

assign mdestReg = dut.mdestReg;

assign mr = dut.mr;

assign mqb = dut.mqb;

assign wwreg = dut.wwreg;

assign wm2reg = dut.wm2reg;

assign wdestReg = dut.wdestReg;

assign wr = dut.wr;

assign wdo = dut.wdo;

assign qa = dut.out_qa;

assign qb = dut.out_qb;

assign fwda = dut.fwdaw;

assign fwdb = dut.fwdbw;

assign MUXaw = dut.MUXaw;

assign MUXbw = dut.MUXbw;

assign qa = dut.out_qa;

assign qb = dut.out_qb;

initial begin

clock = 0;

forever #1 clock = ~clock;

end

initial begin

$monitor("%t | PC=%h | EX/MEM: mr=%h mqb=%h mdest=%d | MEM/WB: wr=%h wdo=%h wdest=%d",

$time, pc, mr, mqb, mdestReg, wr, wdo, wdestReg, MUXaw, MUXbw);

end

initial begin

#200 $finish;

end

endmodule

# Waveform

# Design Schematic

# I/O Planning

Floor planning

Proof of Bitstream and Implementation:

EXTRA CREDIT